Załącznik nr 2 do zapytania ofertowego

SZCZEGÓŁOWY OPIS PRZEDMIOTU ZAMÓWIENIA (SPECYFIKACJA TECHNICZNA) zwany dalej „SOPZ”

- Oferowany sprzęt musi spełniać wszystkie parametry określone w SOPZ oraz musi być fabrycznie nowy i nieużywany.

- Oferowany sprzęt musi być dostarczony Zamawiającemu w oryginalnych opakowaniach.

- Oferowany sprzęt musi być sprawny technicznie, wolny od wad, pochodzić z legalnego źródła i objęty standardowym pakietem usług gwarancyjnych zawartych w cenie urządzenia.

- Każdy ze sprzętów musi być kompatybilny z siecią i sprzętem posiadanym przez Zamawiającego dlatego też przed złożeniem oferty konieczna jest konsultacja z firmą informatyczną obsługującą RDOŚ w Rzeszowie. Kontakt: mceltner@vanona.pl

|  | Lp. | Przedmiot zamówienia | Ilość |  | Opis |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Przełącznik zarządzany | 2 |  | - Porty: 24 x 10/100/1000Mbps, 4 x 10G SFP+ - Komplet wkładek SFP+ 10GbE - Port RS232 lub USB do zarządzania - Montowany w szafie 19”, wysokość 1U - Stackowalny, stos zarządzany za pomocą jednego adresu IP, Przewód DAC SFP+ do stackowania. - Obsługa protokołówIEEE 802.3 10BASE-T Ethernet, IEEE 802.3u 100BASE-TX Fast Ethernet, IEEE 802.3ab 1000BASE-T Gigabit Ethernet, 802.3ae 10 GbE, IEEE 802.3x Flow Control for Full-Duplex Mode, Auto-negotiation - Minimum 16000 adresów MAC - Switch fabric capacity min. 128Gbps - Forwarding rate min. 95.24Mpps - Warstwa L2: • MAC Address Table: Up to 16,384 • Flow Control • 802.3x Flow Control • HOL Blocking Prevention • Jumbo Frame up to 9,216 Bytes • IGMP Snooping • IGMP v1/v2 Snooping • IGMP v3 awareness • Supports 512 IGMP groups • Supports 128 static multicast addresses • IGMP per VLAN • Supports IGMP Snooping Querier • Host-based IGMP Snooping Fast Leave • MLD Snooping • Supports MLD v1/v2 awareness • Supports 512 groups • Supports 128 Static Multicast Addresses • Per VLAN MLD Snooping • Host-based MLD Fast Leave • MLD Snooping Querier • Spanning Tree Protocol • 802.1D STP • 802.1w RSTP • 802.1s MSTP • Loopback Detection v4.07 • 802.3ad Link Aggregation • Max. 32 groups per device/8 ports per group • Port Mirroring • Support 4 mirroring groups • One-to-One, Many-to-One • Supports Mirroring for Tx/Rx/Both • Multicast Filtering • Forwards all unregistered groups • Filters all unregistered groups • Ethernet Ring Protection Switching (ERPS) - VLAN • 802.1Q Tagged VLAN • 4K VLAN Groups • Configurable VID: 0~4094 • GVRP • Asymmetric VLAN • Auto Voice VLAN • Auto Surveillance VLAN 2.1 • MAC-based VLAN • Protocol-based VLAN - Warstwa L3 • ARP 256 Static ARP, Supports Gratuitous ARP • IPv6 Neighbour Discovery (ND) • 16 IP interfaces • Default Routing • Static Routing: 64 IPv4 Static Route Entries, 32 IPv6 Static Route Entries - Access Control List (ACL) - ACL based on • 802.1p priority • VLAN • MAC address • Ether type • IP address • DSCP • Protocol type • TCP/UDP port number • DSCP of IPv6 Traffic Class • IPv6 flow label - ACL Actions • Permit • Deny • Max. 256 access list • Max. 768 rules • Single or multiple ports (each rule) • Time-based ACL • ACL Statistics - Security • Port Security • Supports up to 128 MAC addresses per port • Broadcast/Multicast/Unicast Storm Control • Dynamic ARP Inspection ARP Spoofing Prevention • Max. 64 entries • SSH • Supports v2 • Supports IPv4/IPv6 • BPDU Attack Protection • DoS Attack Prevention • SSL • Supports v1/v2/v3 • Supports IPv4/IPv6 • Traffic Segmentation • IP-MAC-Port Binding • DHCP snooping • IP Source Guard • Dynamic ARP Inspection • IPv6 DHCP Guard • IPv6 RA Guard • IPv6 Snooping • IPv6 Source Guard • IPv6 ND Inspection - AAA • Compound Authentication • 802.1X Port and MAC-based Authentication • Supports RADIUS and Local Server • Supports EAP, OTP, TLS, TTLS, PEAP • Web-based Access Control (WAC) • Port-based Access Control • Host-based Access Control • Dynamic VLAN Assignment • Guest VLAN • RADIUS and TACACS+ authentication for switch access • RADIUS and TACACS+ accounting • MAC-based Access Control (MAC) • Port-based Access Control • Host-based Access Control • Dynamic VLAN Assignment • Japan Web-based Access Control (JWAC) • Port-based Access Control • Host-based Access Control • Dynamic VLAN Assignment - Management • Command Line Interface (CLI) • Telnet Server • TFTP Client • IPv6 Neighbor Discovery • Configurable MDI/MDIX • SNMP • Supports v1, v2c, v3 • SNMP Trap • System Log • Max. 10,000 log entries • Debug command • Multiple images • Surveillance mode • DHCP Client • D-Link Network Assistant support • SNTP • ICMPv6 • IPv4/v6 Dual Stack • DHCP Auto Configuration • RMON v1 • LLDP, LLDP-MED • DHCP relay • Web-based GUI • NTP • Telnet client (supports CLI only) • PD-Alive (PoE models only) - Gwarancja Dożywotnia gwarancja Lifetime-Limited |  |  |
|  | 2 | Moduł SFP+ 10Gbps LC Duplex | 2 |  | Wkładka SFP+ 10GBs do QNAP TS1231XU-RP |  |  |
|  | 3 | Patchcord LC Duplex | 6 |  | LC Duplex 3m |  |  |
|  | 4 | Patchcord UTP kat. 5e | 20 |  | UTP kat.5e kabel sieciowy LAN 2x RJ45 linka szary 1,5m |  |  |
|  | 5 | Patchcord UTP kat. 5e | 20 |  | UTP kat.5e kabel sieciowy LAN 2x RJ45 linka niebieski 3m |  |  |
|  | 6 | Patchcord UTP kat. 5e | 20 |  | UTP kat.5e kabel sieciowy LAN 2x RJ45 linka żółty 1,5m |  |  |